| DDR3L for every need: U-DIMM/VLP, U-DIMM/R-DIMM/VLP, R-DIMM/SO-DIMM |  |
| --- | --- |
| Premier Series DDR3L memory modules offer multiple form factors for various installation needs. All run at a reduced 1.35V compared to 1.5V on standard DDR3, with speeds up to 1600MHz. Whether in standard U-DIMM, very low profile modules (VLP), or small outline modules (SO-DIMM) for notebooks, all implement cutting edge technology that enables remarkable memory quality, stability, and reliability. All modules are produced in accordance with JEDEC and RoHS design and production standards, guaranteeing compatibility with every major computing platform. Each is also made with environmental protection and preservation in mind, and includes a lifetime ADATA warranty. |  |
| Features | U-DIMM VLP U-DIMM R-DIMM VLP R-DIMM SO-DIMM |  |
| - High speed up to 1600MHz - Transfer bandwidths up to 12.8GB/s - Energy efficient: save up to 10% compared to DDR3 - 6-layer PCB provides improved signal transfer and system stability |  |  |

Specifications

| Type | U-DIMM | VLP U-DIMM | R-DIMM | VLP R-DIMM | SO-DIMM |
| --- | --- | --- | --- | --- | --- |
| Form factor | Standard (1.18" height) | Standard (0.7" height) | Standard (1.18" height) | Very low profile (0.7" height) | Standard (1.18" height) |
| Suitable for | Desktop | Desktop / Thin client | Enterprise servers / data centers | Blade servers | Notebook |
| Interface | 240-pin | 240-pin | 240-pin | 240-pin | 204-pin |
| Capacity | 2GB/4GB/8GB | 4GB/8GB | 4GB/8GB/16GB | 4GB/8GB | 2GB/8GB |
| Rank | Single / Dual | Single / Dual | Single / Dual | Single / Dual | Single / Dual |
| Speed | Up to 1600MHz |  |  |  |  |
| VDD voltage | 1.35V |  |  |  |  |
| CAS latency | CL11 |  |  |  |  |
| Refresh | Auto-refresh / self-refresh |  |  |  |  |
| Operating temperature | 0°C-85°C |  |  |  |  |
| RoHS | Yes |  |  |  |  |
| Standard | JEDEC |  |  |  |  |
| Warranty | Limited lifetime warranty |  |  |  |  |

Ordering Information

| Product | Capacity | Rank | Voltage | Data rate | Speed | CL | Part number |
| --- | --- | --- | --- | --- | --- | --- | --- |
| U-DIMM | 2GB | Single | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDU160022G11 |
|  | 4GB | Single | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDU1600W4G11 |
|  | 8GB | Dual | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDU1600W8G11 |
| VLP U-DIMM | 4GB | Single | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDX1600W4G11 |
|  | 8GB | Dual | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDX1600W8G11 |
| R-DIMM | 4GB | Single | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDR1600W4G11 |
|  | 8GB | Dual | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDR1600W8G11 |
|  | 16GB | Dual | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDR1600Y16G11 |
| VLP R-DIMM | 4GB | Single | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDV1600W4G11 |
|  | 8GB | Dual | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDV1600W8G11 |
| SO-DIMM | 2GB | Single | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDS160022G11 |
|  | 8GB | Dual | 1.35V | 1600MT/s | PC3L-12800 | CL = 11 | ADDS1600W8G11 |